# Task 6: Backend Flow Bring-Up with 100 MHz Performance Target

Deadline: 2 Days (EOD Day-2)
Tools: Synopsys ICC2, Star-RC, PrimeTime
Scope: Placement → Routing → Extraction → STA (Flow Validation Task)

## Objective

The objective of this task is to set up and validate a complete backend flow capable of supporting a 100 MHz design target (use same design which you used for floorplan), using industry-standard tools and handoffs.

This task focuses on:

- Correct tool flow setup

- Correct file formats and paths

- Clean handoff between tools

- Basic timing validation

⚠️ This is NOT a signoff-quality closure task.
The goal is to prove that the end-to-end backend flow works.

## Performance Target

- Target Frequency: 100 MHz

- Clock Period: 10 ns

Timing must be checked using post-route parasitics (SPEF).

## Flow to Be Implemented

Participants must implement the following exact sequence:

ICC2 (Placement + Routing)

↓

Star-RC (SPEF Extraction)

↓

PrimeTime (Post-Route STA @ 100MHz)

## Phase-1: ICC2 Placement & Routing

### 1. Inputs to ICC2

- Synthesized netlist (DC_TOPO output)

- Floorplan database from previous task

- REFERENCE standard cell LEFs

- REFERENCE technology LEF

- Timing libraries (.lib)

- IO pad LEFs

### 2. Tasks in ICC2

Participants must:

- Perform standard cell placement

- Perform clock tree synthesis (basic CTS)

- Perform signal routing

- Generate a post-route database

⚠️ No ECO, no timing optimization required
⚠️ Clean run is more important than perfect WNS

### 3. Outputs from ICC2

- Routed DEF

- Post-route netlist

- GDS (optional but recommended)

- Timing reports (pre-extraction)

## Phase-2: Parasitic Extraction Using Star-RC

### 4. Star-RC Setup

Participants must:

- Configure Star-RC tech files

- Point to:

  - Routed DEF

  - Post-route netlist

  - REFERENCE extraction tech

### 5. Extraction Objective

- Generate SPEF for:

  - Entire design

  - Clock nets included

### 6. Outputs

- design.spef

- Star-RC extraction log

- Extraction summary report

## Phase-3: Post-Route STA Using PrimeTime

### 7. PrimeTime Setup

Participants must:

- Read:

  - Post-route netlist

  - Extracted SPEF

  - Timing libraries

- Define clock:

  - create_clock -period 10

### 8. STA Checks (Mandatory)

- Setup timing

- Hold timing

- Clock skew

- Worst negative slack (WNS)

- Total negative slack (TNS)

⚠️ Timing closure is not mandatory, but:

- Design must analyze cleanly

- No missing constraints

- No broken paths

### 9. Outputs

- Setup timing report

- Hold timing report

- Clock report

- Summary timing report

## Documentation Requirement (Very Important)

This task is as much about flow clarity as timing.

Participants must clearly document:

- Directory structure

- Tool versions

- Exact commands used

- Paths to all inputs

- Handoff points between tools

This documentation should allow another engineer to reproduce the flow.

## Deliverables (Mandatory)

Task_Backend_100MHz_Flow/

├── icc2/

│   ├── scripts/

│   ├── reports/

│   └── outputs/

├── star_rc/

│   ├── scripts/

│   ├── spef/

│   └── logs/

├── primetime/

│   ├── scripts/

│   └── reports/

├── constraints/

│   └── clocks.sdc

└── README.md

### README Must Include:

- 100 MHz target explanation

- Clock definition used

- ICC2 → Star-RC → PT handoff explanation

- Any issues faced and how they were resolved

## Evaluation Criteria

| Area | Priority |
| --- | --- |
| End-to-end flow correctness | Extremely High |
| Proper tool handoff | Extremely High |
| SPEF correctly read in PT | Very High |
| STA reports generated | Very High |
| Documentation clarity | Extremely High |