# Task4: Full Management SoC DV Validation on SCL-180 (POR-Free Design)

Duration: 2 Days (Today + Tomorrow)
Deadline: End of Day Tomorrow (11:59 PM IST)
Submission: Mandatory GitHub documentation + evidence
Tools: Synopsys VCS, DC_TOPO, SCL-180 PDK

Reference DV tests link - https://github.com/efabless/caravel/tree/main/verilog/dv/caravel/mgmt_soc

## Objective

The objective of this task is to prove that our POR-free RTL is production-ready by:

- Running all Management SoC (mgmt_soc) DV tests originally written for Caravel

- Using a new SCL-180 netlist generated via DC_TOPO

- Running the same tests twice:

  - Phase-A: Using RTL SRAM models

  - Phase-B: Using DC_TOPO synthesized SRAM

- Demonstrating that behavior is identical across:

  - RTL simulation

  - GLS with RTL SRAM

  - GLS with synthesized SRAM

This task confirms that:

- Removing POR is safe

- Reset architecture is correct

- SRAM integration is robust

- RTL has no hidden power-up assumptions

## Background (Why this task exists)

The original Caravel mgmt_soc DV suite validates:

- Housekeeping SPI

- GPIO configuration and transfer logic

- User project control

- Storage interfaces

- IRQ behavior

These tests do not rely on POR — they rely on:

- External reset (RSTB)

- Correct pad behavior

- Proper reset distribution

Running these tests on SCL-180 with a POR-free netlist is a strong industry-grade validation.

## Scope of Work

Participants must run all Management SoC DV tests using:

- SCL-180 PDK

- Synopsys VCS

- DC_TOPO-generated netlist

### mgmt_soc DV coverage includes:

- hkspi

- gpio

- mprj_ctrl

- storage

- irq

## Phase-1: Environment Setup (Mandatory)

### 1. Toolchain Requirements

- Synopsys VCS (functional + GLS)

- Synopsys DC_TOPO

- SCL-180 standard cell libraries

- SCL-180 IO models

## Phase-2: POR-Free Netlist Generation

### 2. RTL Preparation

- Confirm:

  - No dummy_por, simple_por, or POR logic exists

  - Single external reset pin (reset_n / RSTB)

- Reset must be driven only from testbench

### 3. DC_TOPO Synthesis (Baseline Netlist)

- Synthesize full SoC using DC_TOPO

- SRAM remains RTL model in this phase

- Generate:

  - Synthesized netlist

  - Area, timing, and power reports

Deliverable:

- netlist_rtl_sram/

- DC reports + logs

## Phase-3: Management SoC DV – Run-1 (RTL SRAM)

### 4. VCS Functional + GLS with RTL SRAM

For each mgmt_soc DV test:

- hkspi

- gpio

- mprj_ctrl

- storage

- irq

Participants must:

- Use VCS

- Include:

  - DC_TOPO netlist

  - RTL SRAM models

  - SCL-180 standard cell models

- Drive:

  - External reset from testbench

  - Power rails as per DV

### Expected Results:

- All tests pass

- No X-propagation after reset

- Identical behavior to reference DV intent

Deliverables (per test):

- Simulation log

- FSDB/VPD waveform

- Screenshot of key pass condition

## Phase-4: SRAM Synthesis (Critical Validation Phase)

### 5. DC_TOPO SRAM Synthesis

- Identify SRAM RTL used in SoC

- Synthesize SRAM using DC_TOPO

- Generate:

  - SRAM gate-level netlist

  - Timing reports

No behavioral SRAM allowed in this phase.

Deliverable:

- sram_dc_topo/

- Netlist + reports

## Phase-5: Management SoC DV – Run-2 (Synthesized SRAM)

### 6. VCS GLS with Synthesized SRAM

Re-run all mgmt_soc DV tests again, but now with:

- DC_TOPO SoC netlist

- DC_TOPO SRAM netlist

- SCL-180 standard cell models

### Expected Results:

- Same behavior as Phase-3

- No new X-states

- No reset-related failures

- No memory corruption

This is the final proof of RTL correctness.

Deliverables (per test):

- GLS log

- FSDB/VPD waveform

- Screenshot showing correct operation

## Phase-6: Engineering Documentation (Mandatory)

### 7. Final Report (Most Important)

Create a document titled:

“Management SoC DV Validation on SCL-180 (POR-Free Architecture)”

It must include:

- Why POR was removed

- Why mgmt_soc DV does not require POR

- Reset strategy used

- Differences between:

  - RTL SRAM

  - Synthesized SRAM

- Observations from both runs

- Issues faced and how they were resolved

This document should read like a design sign-off note, not a lab report.

## Submission Structure

Task_MgmtSoC_SCL180_Final/

├── netlist_rtl_sram/

├── sram_dc_topo/

├── dv_run_rtl_sram/

│   ├── hkspi/

│   ├── gpio/

│   ├── mprj_ctrl/

│   ├── storage/

│   └── irq/

├── dv_run_synth_sram/

│   ├── hkspi/

│   ├── gpio/

│   ├── mprj_ctrl/

│   ├── storage/

│   └── irq/

├── reports/

└── README.md

## Evaluation Criteria

| Area | Weight |
| --- | --- |
| Correct tool usage (VCS + DC_TOPO) | High |
| POR-free correctness | Very High |
| DV pass completeness | Very High |
| Synthesized SRAM validation | Extremely High |
| Documentation clarity | Extremely High |

Low-effort or partial submissions will be rejected.