Task3 - Removal of On-Chip POR and Final GLS Validation (SCL-180)

Deadline: Tomorrow EOD
Submission: Mandatory GitHub documentation + evidence
Tools: Synopsys VCS, DC_TOPO, SCL-180 PDK
Status: Research-driven, industry-style task

Objective

The objective of this task is to formally remove the on-chip Power-On Reset (POR) from the VSD Caravel-based RISC-V SoC and prove—using design reasoning, pad analysis, synthesis, and gate-level simulation—that an external reset-only strategy is safe and correct for SCL-180.

By the end of this task, participants must demonstrate:

- A POR-free SoC RTL that relies only on an external reset pin

- A clear technical justification, based on pad behavior and power assumptions, for why this is safe

- A clean DC_TOPO synthesis

- A final VCS-based GLS with SCL-180 standard cells

- Industry-quality documentation explaining why POR was removed—not just how

Context (Why this task exists)

In earlier flows, a behavioral dummy_por was used to model reset sequencing. However:

- Behavioral PORs are not synthesizable

- True PORs are analog macros, not RTL

- In sane pad libraries (e.g., SCL-180), pads and reset pins are usable immediately after power-up

- Therefore, a single external reset pin is architecturally sufficient

This task validates that decision rigorously.

Task Breakdown

Phase-1: POR Dependency Analysis (Research Phase)

1. Study and Document Existing POR Usage

You must:

- Identify where and how dummy_por is used in:

  - vsdcaravel.v

  - Housekeeping logic

  - Reset distribution paths

- Create a short document explaining:

  - What signals porb_h, porb_l, por_l drive

  - Which blocks actually depend on POR versus generic reset

Deliverable:

- Markdown or PDF:
POR_Usage_Analysis.md

2. PAD Library Study (Critical Thinking Section)

Study the SCL-180 I/O pad documentation and answer:

- Does the reset pad require:

  - Internal enable?

  - POR-driven gating?

- Is the reset pin:

  - Asynchronous?

  - Available immediately after VDD?

- Are there documented power-up sequencing constraints that mandate a POR?

This section must explicitly contrast SCL-180 with SKY130, explaining why POR was mandatory in SKY130 but not here.

Deliverable:

- PAD_Reset_Analysis.md

- Include references to pad datasheets or comments from PDK files

Phase-2: RTL Refactoring (Engineering Phase)

3. Remove POR from RTL

You must:

- Completely remove:

  - dummy_por

  - All POR-related signals

- Introduce a single top-level reset pin:

  - reset_n (active-low, external)

- Ensure:

  - All sequential logic resets off this signal

  - No hidden POR assumptions remain

Rules:

- No digital POR

- No counters

- No power-pin edge detection

- Reset behavior must be explicit and visible

Deliverable:

- POR-free RTL committed to GitHub

- Commit message must clearly state:

“Removed on-chip POR; migrated to external reset-only architecture”

Phase-3: Synthesis with DC_TOPO

4. DC_TOPO Synthesis (POR-Free)

Run synthesis using DC_TOPO with:

- SCL-180 standard cell libraries

- Proper reset constraints

- No POR blackboxes

You must generate:

- Synthesized netlist

- Area, timing, and power reports

Key Check:

- No unresolved reset nets

- No floating enable pins

- No inferred latches due to reset removal

Deliverable:

- synthesis/ folder containing:

  - Netlist

  - Reports

  - DC logs

Phase-4: Final Gate-Level Simulation (GLS)

5. VCS-Based GLS (Final Proof)

Run full chip GLS using:

- VCS

- DC_TOPO netlist

- SCL-180 functional cell models

- External reset asserted from testbench

You must show:

- Clean reset assertion and de-assertion

- No X-propagation

- Functional equivalence with RTL behavior

Important:

- Reset must be driven from the testbench

- No internal reset generation logic allowed

Deliverable:

- GLS log

- FSDB/VPD waveform

- Screenshot showing reset release and normal operation

Phase-5: Engineering Justification (Most Important Part)

6. Final Decision Document

Create a clear, technical justification titled:

“Why External Reset Is Sufficient in SCL-180 (No POR)”

This must include:

- Why POR is an analog problem

- Why RTL-based POR is unsafe

- Why SCL-180 pads allow safe external reset

- Risks considered and mitigations

- Comparison with SKY130

This document is non-negotiable and will be evaluated like a design review.

Deliverable:

- POR_Removal_Justification.md

Submission Structure (Mandatory)

Task_NoPOR_Final_GLS/

├── POR_Usage_Analysis.md

├── PAD_Reset_Analysis.md

├── POR_Removal_Justification.md

├── rtl/

├── synthesis/

├── gls/

├── waveforms/

└── README.md

Evaluation Criteria

| Area | Weight |
| --- | --- |
| Technical correctness | High |
| PAD & reset reasoning | Very High |
| Clean synthesis | High |
| Clean GLS | High |
| Documentation clarity | Extremely High |

Superficial “it works” submissions will be rejected.