## Task1 - RISC-V Reference SoC — Functional and GLS Replication (SCL180) on your own IITGN Machine

Deadline: Today EOD
Action: Push results to your individual GitHub repository

## Objective

You must replicate the complete functional simulation and gate-level simulation (GLS) of the vsdcaravel RISC-V SoC using SCL180 PDKs, exactly as demonstrated in the reference repository:

Reference:
https://github.com/vsdip/vsdRiscvScl180/tree/iitgn

This task ensures that you understand the SoC structure, RTL integration, simulation flow, synthesis readiness, and overall bring-up flow required for the SCL180 tapeout process.

## Task Breakdown

### 1. Study the Reference Repository

Review the structure and purpose of each folder:
rtl/, dv/, synthesis/, gl/, gls/, images/.

Understand the top module (vsdcaravel.v), integration of VexRiscv, RAM128, POR, housekeeping, IO pads, and the hkspi block.

### 2. Run Functional (RTL) Simulation

Inside dv/hkspi/:

- Set paths in the Makefile:

  - GCC_Path → Your RISC-V GCC

  - scl_io_PATH → SCL IO models

- Run:

- make clean

- make

- vvp hkspi.vvp

- gtkwave hkspi.vcd hkspi_tb.v

- Expected results:

  - Successful console output

  - Clean waveform confirming correct hkspi operation

Upload the VCD, logs, and waveform screenshots to your repo.

### 3. Run Gate-Level Simulation (GLS)

Inside gls/:

- Modify the synthesized netlist:

  - Remove black-box entries for: dummy_por, RAM128, housekeeping

  - Add the following at the top:

  - `include "dummy_por.v"

  - `include "RAM128.v"

  - `include "housekeeping.v"

- In vsdcaravel.v, replace 1'b0 with vssa for power pin wiring.

- Update gls/Makefile:

  - Add paths for SCL PDK, RISC-V GCC, and IO libraries.

- Run GLS:

- make clean

- make

- vvp hkspi.vvp

- gtkwave hkspi.vcd hkspi_tb.v

- Expected results:

  - GLS waveform closely matching RTL waveform

  - No unknown states on critical paths

Upload the netlist, logs, VCD, and waveform screenshots.

### 4. Update Your Repository

Create a folder named Day1_Task_Replication/ containing:

- Functional simulation evidence

- GLS evidence

- A README summarizing:

  - Steps taken

  - System details

  - Issues encountered and resolved

## Deliverables Checklist

| Task | Deliverable |
| --- | --- |
| Functional Simulation | Log, VCD, waveform screenshot |
| GLS | Netlist, log, VCD, waveform screenshot |
| Repository Update | README + evidence folder |

## Deadline

Today – 11:59 PM
Submissions without evidence will be considered incomplete.